# Phoenix Topdown Profiler

## Overview

phoenix_topdown_profiler.py is an end-to-end automation wrapper around topdown-tool for collecting and merging:

- CPU Topdown metrics

- CMN (Coherent Mesh Network) telemetry

- DMC Phoenix Frontend telemetry

- DMC Phoenix Backend telemetry

- Optional workload telemetry extracted from workload stdout

The profiler executes a predefined collection schedule that gathers CPU, CMN, and DMC metrics across multiple passes and merges the results into a single unified timeline.

# Dependencies

## Python

Supported:

Python 3.9+

Verify:

python3 --version

## Python Packages

Install:

python3 -m pip install pandas numpy

Required:

pandas
numpy

Used for:

CSV parsing
aggregation
timeline generation
metric normalization

## Topdown Tool

Profiler requires:

topdown-tool

Verify:

topdown-tool --version

and:

topdown-tool --hwplatform phoenix --probe-list

# Collection Methodology

The profiler performs:

CPU Analysis
    ↓
CMN Analysis
    ↓
DMC FE Analysis
    ↓
DMC BE Analysis

This follows the workflow:

CPU symptom
    ↓
CMN traffic location
    ↓
DMC memory controller validation

# Running Collection

## Single Frame

rm -rf phoenix_out

python3 phoenix_topdown_profiler.py \
  --frames 1 \
  --core 4 \
  --cmn-indices 0,1 \
  --cmn-clock-ghz 2.0 \
  --workload "taskset -c 4 /root/bw_mem_init_once -P 1 -N 1000 -W 2 1024m rd"

## Multi-frame Timeline

Example:

rm -rf phoenix_out

python3 phoenix_topdown_profiler.py \
  --frames 10 \
  --core 4 \
  --cmn-indices 0,1 \
  --cmn-clock-ghz 2.0 \
  --workload "taskset -c 4 /root/bw_mem_init_once -P 1 -N 1000 -W 2 1024m rd"

Produces a time series across frames.

# Re-parsing Existing Data

No recollection required.

python3 phoenix_topdown_profiler.py \
  --parse-only \
  --out phoenix_out \
  --cmn-clock-ghz 2.0

# Output Directory

Final outputs:

phoenix_out/
└── merged/
    ├── README.txt
    ├── timeline_unified.csv
    └── timeline_long_raw_devices.csv

# timeline_unified.csv

Primary analysis file.

Columns:

frame
slot
slot_name
domain
dmc_subdomain
metric
effective_unit
preferred_value

Example:

0,12,slot12_dmc_be_queue_bw,DMC,BE,dmc_dram_channel_read_data_percentage,%,3.91

What we cover!

| CPU | Metrics / groups | What it covers |
| --- | --- | --- |
| Topdown L1 | retiring, frontend_bound, backend_bound, bad_speculation | High-level CPU bottleneck split |
| General | ipc, cycles/instructions style metrics | Overall CPU efficiency |
| Backend | backend_* metrics | Core vs memory backend pressure |
| Cycle accounting | frontend_stalled_cycles, backend_stalled_cycles | Where cycles are being lost |
| Cache miss ratios | l1d_*, l2_*, ll_* ratios | L1/L2/SLC effectiveness |
| TLB | itlb_*, dtlb_* | Instruction/data translation pressure |
| Operation mix | load/store/branch percentages | Workload instruction behavior |

| CMN | Metrics / groups | What it covers |
| --- | --- | --- |
| Requestor bandwidth | cmn_bw_rnf, cmn_bw_rni, cmn_bw_rnd | CPU, IO, and device-originated traffic |
| Completer bandwidth | cmn_bw_snf | DRAM-facing CMN bandwidth |
| Cross-chip bandwidth | cmn_bw_ccg_c2c_requestor, cmn_bw_ccg_c2c_destination | Remote / C2C traffic |
| HNS data traffic | cmn_hns_txdat_rate | Data response traffic through HNS |
| HNS queueing | cmn_hns_pocq_avg_occupancy | HNS pending queue pressure |
| HNS retry/throttle | cmn_hns_pocq_retry_ratio, cmn_hns_memcntrl_retry_ratio, cmn_hns_memcntrl_cbusy_req_throttle_rate | Memory-side pressure seen at HNS |
| HNS CBusy | cmn_hns_cbusy0/1/2/3_* | HNS congestion/busy state |
| CCG ingress/egress | cmn_ccg_rx*, cmn_ccg_tx* | Cross-chip request/data/response traffic |
| CCG retry | cmn_ccg_rxrsp_retry_rate, cmn_ccg_txrsp_retry_rate | CCG congestion/retry indication |
| XP traffic | cmn_xp_txdat_*port_rate | Mesh directional traffic |

| DMC | Metrics / groups | What it covers |
| --- | --- | --- |
| FE command mix | chi_rd_percentage, chi_wr_percentage, chi_readnosnp*, chi_writenosnp*, chi_prefetchtgt_percentage | CHI request type distribution entering DMC |
| FE read-data busy | dmc_fe_rdata_cbusy0_pct, dmc_fe_rdata_cbusy1_pct, dmc_fe_rdata_cbusy2_pct, dmc_fe_rdata_cbusy3_pct | DMC read-data CBusy state, renamed from tool-native busy categories |
| FE retry | chi_retry_percentage, chi_rd_retry_percentage, chi_wr_retry_percentage, no-retry variants | DMC FE retry pressure |
| FE zero credits | chi_request_zerocredits, chi_response_zerocredits, chi_read_data_zerocredits | CHI link-credit starvation |
| FE buffers | chi_read_buffer_occupancy, chi_write_buffer_occupancy | DMC frontend queue/buffer pressure |
| BE queue | dmc_command_queue_occupancy, dmc_read_cmdq_alloc/dealloc, dmc_write_cmdq_alloc/dealloc | Backend command queue utilization |
| BE DRAM utilization | dmc_dram_channel_read_data_percentage, dmc_dram_channel_write_data_percentage, dmc_dram_channel_data_util_percentage, dmc_dram_channel_remainder_percentage | DRAM channel read/write/idle split |
| BE DRAM commands | dram_read_rate, dram_write_rate, dram_total_cmd_rate, activate/precharge/refresh rates | DRAM command mix |
| BE low power | dmc_dram_lp_*_percentage | DRAM low-power residency |

| Symptom | CPU Check | CMN Check | DMC Check | Interpretation |
| --- | --- | --- | --- | --- |
| High latency | backend_mem_bound | cmn_hns_pocq_avg_occupancy, cmn_hns_memcntrl_retry_ratio | dmc_command_queue_occupancy | Requests are waiting in memory subsystem |
| High memory stalls | backend_mem_bound | cmn_hns_memcntrl_cbusy_req_throttle_rate | dmc_dram_channel_data_util_percentage | Memory bandwidth or queue saturation |
| Poor IPC | ipc, backend_bound, frontend_bound | Follow CMN path only if backend-bound | Follow DMC path only if memory-bound | CPU bottleneck classification |
| Frontend issue | frontend_bound | Ignore CMN initially | Ignore DMC initially | Fetch/decode bottleneck |
| Core execution issue | backend_core_bound | Ignore CMN | Ignore DMC | Execution resources, dependencies, scheduling |
| Translation issue | itlb_*, dtlb_* | Ignore CMN | Ignore DMC | TLB pressure |